IR Remote Control

Functional Requirements

1. The IR remote control shall transmit infrared control signals to compatible consumer electronic devices.

1.1 The IR remote control shall encode user commands into a digital IR transmission format.

1.2 While a command button remains pressed, the IR remote control shall retransmit the IR code at intervals of ≤ 120 ms.

1.3 When a command button is pressed, the IR remote control shall transmit the corresponding IR code.

2. The IR remote control shall provide a user interface for manual command input.

2.1 The IR remote control shall provide a minimum of 20 physical buttons for device control functions.

2.2 The IR remote control shall provide tactile feedback for each button press.

2.3 The IR remote control shall provide visual indication using an LED when an IR signal is transmitted.

3. The IR remote control shall support multi-device control through programmable command codes.

3.1 The IR remote control shall allow the user to configure command codes for at least one target device.

3.2 The IR remote control shall store the configured command codes in non-volatile memory.

3.3 The IR remote control shall retain the stored command codes when power is removed or batteries are replaced.

4. The IR remote control shall monitor and manage its power status.

4.1 The IR remote control shall detect when the battery voltage falls below the minimum operating threshold.

4.2 If the battery voltage is below the minimum operating threshold, then the IR remote control shall provide a low-battery indication.

4.3 If the battery voltage is below the minimum operating threshold, then the IR remote control shall prevent IR transmission.

Non-functional Requirements

1. Performance

1.1 The IR remote control shall start transmitting an IR signal within 100 ms after a valid button press.
1.2 The IR remote control shall achieve a transmission range of at least 8 m under indoor conditions.
1.3 The system shall successfully deliver commands with a reception success rate of ≥ 95% at the specified operating distance.

2. Power Efficiency

2.1 The IR remote control shall operate for at least 6 months under normal usage.
2.2 The standby current consumption shall be ≤ 20 µA.
2.3 The operating current during IR transmission shall be ≤ 100 mA.

3. Reliability and Availability

3.1 The IR remote control shall operate correctly in an ambient temperature range of 0°C to 45°C.
3.2 The push buttons shall support at least 100,000 press cycles without functional failure.